Specifications

P2.5 OUTDOOR LED POSTER DISPLAY  WITH DIE CASET ALUMINUM  CABINET TECHNICAL PARAMETERS-1920
	Unit Module
	Item
	Parameter

	
	Model Number
	P2.5

	
	Module size
	320mm*160mm

	
	Pixel pitch
	2.5mm

	
	Pixels Density
	160000/m2

	
	Pixel configuration
	1R1G1B

	
	LED  Package mode
	SMD1515

	
	 protecting
	true

	
	Pixel resolution
	128dots(W)*64dots(H)

	
	Max Power
	22.5W

	
	Weight
	0.24kg

	
	Drive type
	Constant drive

	
	Scan mode
	1/43 scan

	
	Port type
	HUB75-E

	
	Brightness of white balance
	≥5000cd/m2


	Cabinet
	Cabinet Dimension
	1920mm*640mm

	
	Monomer Module Number
	24

	
	Cabinet material
	Iron box body

	
	Power supply
	5V/40A*1 pcs

	
	Thermal discharge
	Self cooling

	
	Quantity of cabinets
	4

	
	Pixel per cabinet
	344*1032

	
	Seamless splicing
	true

	
	Max. splicing
	10 sets

	
	Best viewing distance
	≥2M

	
	Best viewing angle
	160°(W) 140°(H)

	
	Temperature
	storage:-35℃～+85℃

working:-20℃～+50℃

	
	Relative humidity
	10%-95%

	
	Thickness
	60mm

	
	Weight
	55KG

	Power
	Working voltage
	AC110/220V±10%（Optional）

	
	Average Power consumption
	145W

	
	Max.Power consumption
	≤430W

	
	Current
	≤20mA(single LED)

	Control system
	Communication  method
	WIFI / LAN / HDMI

	
	Control method
	Syn/Asy

	Main technical

parameter
	Driving device
	Constant drive IC

	
	Refresh frequency
	≥3840HZ

	
	Grey scale/colors
	4096gray level/16.7M colors

	
	MTBF
	≥10000hours

	
	Power switch
	Manual switch


